Raspberry Pi Setup

- Setting up raspberry pi  for the first time using USB OS.

- Download  the following softwares :

  - Advanced IP Scannerd

  - Mobaxterm

  - Win32diskimager

- Install OS from Raspberrypi.org : Use raspberian  OS. Download the software ( .img ) file and use Win32diskimage and write the .img in the USB drive .

- Go to config.txt , set at the beginning and save the file:

program_usb_boot_mode=1

- Go to the F:\ Drive ( USB Drive ) and give the echo > ssh in order to enable ssh in raspberry  :

F:\> echo > ssh

- Go to   Control Panel\ Network and Internet\ Network Connections :

- Select Ethernet connections , select IPV4 and set the properties

Static IP set is 10.0.0.7 in Ethernet Connections.

- Go to cmdline.txt  in F:\

At the end of the file, cmdline.txt , give a space and set :

ip=10.0.0.6  and save it.

- Connect the Raspberry Pi as shown below :

- USB drive to usb port of  raspberry pi

- Connect Ethernet cable to the computer

- Connect mini USB port to the power source.

- Connect to Mobaxterm as follows 10.0.0.6 ( Note that Port is set in cmdline.txt ) :

- Login to 10.0.0.6 :

User id : pi

Password : raspberry

- Password change :

passwd is :  john!john

Command for BIOS like setup : configure WIFI , enable UI at boot.

sudo raspi-config

Go to Boot option Desktop with ui login

Go to Networking Option  Network Option  Wifi

Redmij / zxcasdqwe123

sudo shutdown -h now

sudo reboot

https://www.elecrow.com/wiki/index.php?title=HDMI_Interface_5_Inch_800x480_TFT_Display

LCD setup :

Step1: Modify your config.txt file

Tips: If you use SSH to control Pi,please skip this step.

Insert the SD card to your Windows/Mac PC. Find the config.txt in the SD`s root and open it. Then add the following code in the end.

# --- added by elecrow-pitft-setup  ---

hdmi_force_hotplug=1

max_usb_current=1

hdmi_drive=1

hdmi_group=2

hdmi_mode=1

hdmi_mode=87

hdmi_cvt 800 480 60 6 0 0 0

dtoverlay=ads7846,cs=1,penirq=25,penirq_pull=2,speed=50000,keep_vref_on=0,swapxy=0,pmax=255,xohms=150,xmin=200,xmax=3900,ymin=200,ymax=3900

display_rotate=0

# --- end elecrow-pitft-setup  ---

git clone https://github.com/Elecrow-keen/Elecrow-LCD5.git

cd Elecrow-LCD5

sudo ./Elecrow-LCD5

sudo apt-get install -y xinput-calibrator

/etc/X11/xorg.conf.d/99-calibration.conf

- Setting up raspberry pi  for the first time using USB OS and 5 inch screen.

Once all the setup in step I is complete, monitor can be connected and tested.

- Connect the HDMI cable and the USB cable to Monitor

- Connect other end of the the HDMI cable and the USB cable of  Monitor to Raspberry Pi

- Connect Raspberry Pi’s  USB OS as shown below

- Connect Raspberry Pi to Keyboard , mouse and finally power the Raspberry PI through USB cable.

- Raspberry Pi booting up:

- Login to Raspberry Pi using the user id / password configured in Step I:

Since Raspberry PI is setup for WIFI Connection using  “sudo raspi-config” in  Step I, connecting to internet should not be problem.

- Accessing raspberry pi  Desktop using Windows Computer

- Connect to Raspberry Pi using MobaXterm

Enter the following commands :

sudo apt-get update

sudo apt-get upgrade

### ENABLING VNC SERVER AT THE COMMAND LINE

You can enable VNC Server at the command line using raspi-config:

sudo raspi-config

Now, enable VNC Server by doing the following:

- Navigate to Interfacing Options.

- Scroll down and select VNC > Yes.

- Connect to Raspberry Pi using MobaXterm

Install  VNC Viewer from Real VNC.

Connect using IP : 10.0.0.6

User id : pi

Password : john!john.

Appendix  Commands:

sudo apt-get update

sudo apt-get upgrade

Tried out commands

Following steps are NOT required :

sudo apt-get install tightvncserver

Password ( only 8 characters allowed ): john!joh

vncserver :1

sudo nano /etc/rc.local

Ctrl X to exit , give Y to save and enter.

Sudo  reboot